# Final Exam Problem 04:  Chapter 05

θ

The rod shown has a length L=4ft, and is pinned at its end O.  The attached spring has a stiffness k=50lb/ft, and an unstretched length lo=2ft.  The spring is unstretched when θ=0°.

- Draw the free body diagram for rod OC.  Do this carefully and pay attention to the various angles!

See free body diagram on the right!

- Calculate the spring force F=k∆l by determining the stretched length l of the spring in the position shown.

Use Law of Cosines on the triangle

A2=B2+C2-2BCcosα
l2=L22+L22-2L22cos45°
l2=4ft22+4ft22-24ft22cos45°
l=2.343ft
F=kl-lo=50lbft2.343-2ft=17.2lb

- Determine the tension T in the cord when the system is in equilibrium at 45°  (Hint:  You’ll need to solve for the spring angle θ!).

Use Law of Sines to find the angle

lsin45°=L2sinθ
θ=sin-1L2sin45°l=sin-12ftsin45°2.343ft=37.1°

Sum the moments about the origin. Let counter-clockwise be the positive direction:

MO=y1Fx+x1Fy+y2Tx+x2Ty=0
0.5Lcos45°Fsinθ+0.5Lsin45°Fcosθ+Lcos45°Tcos60°+Lsin45°Tsin60°=0
cos45°Tcos60°+sin45°Tsin60°=-0.5sin45°Fcosθ-0.5cos45°Fsinθ
Tcos60°+sin60°=-0.5Fcosθ+sinθ
T=-0.517.2bcos37.1°+sin37.1°cos60°+sin60°=-8.82lb

- Determine the reaction force R at point O.

Sum the forces and solve the system

Fx=Rx+Fsinθ+Tcos60°=0
Rx=-Fsinθ-Tcos60°=-17.2lbsin37.1°--8.82lbcos60°=-5.96lb
Fy=Ry+Fcosθ+Tsin60°=0
Ry=-Fcosθ-Tsin60°=-17.2lbcos37.1°--8.82lbsin60°=-6.08lb
R=-5.96i+6.08jlb